# SSP serial interface protocol

| SYNTAX | Variable Definitions | Function | Return / Response |
| --- | --- | --- | --- |
| SSMODE | #### none | Establish serial connection | ! |
| SGAINn | where n = 1, 2, or 3 | Select instrument gain where 1 = 100X 2 = 10X 3 = 1X | ! |
| SIwxyz | where w,x,y,z are integers 0..9 | Set integration time where the sum is z * 10 msec, y * 100 msec, x * 1 second, w * 10 seconds. | ! |
| SCOUNT | none | Start integration | C=y where y = instrument count output i.e. 0 to 65535 |
| SMnnnn | where nnnn = 0000 to 9999 | Start fast mode integration | nnnn where nnnn = output count from 0000 to 9999. |
| SFILTn | #### where n = #### 1,2,3,4,5,or 6 | Move auto filter slider to position n. | ! |
| SHOMEx | #### none | Return auto filter slider to position 1. | ! |
| SVIEWn | #### where n = 0, 1 | Control flip mirror n = 0 to view, n = 1 to take data | ##### none |
| SENDxx | #### None | Terminate serial connection | ##### END |

07.29.2026

/misc/SSP protocol.doc